1000, 2000, 3000, 4000, 4050, 5000, 6000, 7000, 6006E/9006 audio systems

View of 5000C 

	


antenna.

Views of connectors• NOTE:Further information on this can be found in the Owner's Handbook CG 3358.

	[image: image1.jpg]





Socket A

	Pin
	Connection

	1
	Terminal 30, + battery positive 

	2
	Battery ground

	3
	Ignition

	4
	Lighting

	5
	Antitheft control signal

	6
	Terminal 31 battery ground

	7
	Switched positive for antenna

	8
	Not assigned

	Socket BPin
	Connection

	1
	Loudspeaker, front left +

	2
	Loudspeaker, front left -

	3
	Loudspeaker, rear left +

	4
	Loudspeaker, rear left -

	5
	Loudspeaker, front right +

	6
	Loudspeaker, front right -

	7
	Loudspeaker, rear right +

	8
	Loudspeaker, rear right -

	Socket CPin
	Connection

	1
	ACP +

	2
	ACP -

	3
	Parking aid

	4
	Phone mute

	5
	Ground, shielded

	6
	Automatic volume control (AVC)

	7
	Not assigned

	8
	Audio #1, Telematics

	9
	Audio #2, Telematics

	10
	Telematics

	11
	Remote control 1

	12
	Remote control 2

	13
	SCP +

	14
	SCP -

	15.16.17
	Not assigned

	16
	Not assigned

	17
	Not assigned


Audio systems 5000C, 6000CD, new generation

View of 5000C, 2003.75 Mondeo (06/2003-) 

	[image: image2.jpg]





• 

	The table shows the maximum pin assignment.
	Views of connectors 

[image: image3.jpg]



A

Socket A, pins 1 - 16

B

Socket B, pins 1 - 12

C

Socket C, pins 1 - 12

D

Fuse




Socket A

	Pin
	Connection
	Pin
	Connection

	1
	Rear right +
	9
	CAN +

	2
	Front right +
	10
	CAN -

	3
	Front left +
	11
	Keycode ground

	4
	Rear left +
	12
	Ground

	5
	Rear right -
	13
	Alarm

	6
	Front right -
	14
	Lighting

	7
	Front left -
	15
	Battery +

	8
	Rear left -
	16
	Ignition


Socket B

	Pin
	Connection

	1
	Mono +

	2
	Sw A+

	3
	Accessories (AUX) L +

	4
	Accessories (AUX) R +

	5
	Phone mute

	6
	SWC

	7
	Mono -

	8
	SWC ground

	9
	Accessories (AUX) L -

	10
	Accessories (AUX) R -

	11
	Reverse gear

	12
	Automatic volume control (AVC)


Socket C

	Pin
	Connection

	1
	Remote control, rear L +

	2
	Remote control, rear R +

	3
	Mono 2 +

	4
	Subwoofer +

	5
	Accessories (AUX) 2 L

	6
	Ground

	7
	Remote control, rear L -

	8
	Remote control, rear R -

	9
	Mono 2 -

	10
	Subwoofer -

	11
	Accessories (AUX) 2 R

	12
	Not assigned


